SOFTWARE REQUIREMENTS SPECIFICATION

SPAcecraft Status Display And Commanding Software

SPASDACS Nova

| Document No. | SCG-GRCD2-SW-2025-03 |
| --- | --- |
| Version | 2.0 |
| Date | March 2026 |
| Prepared by | GEOSAT RF & Payload Checkout Division-2 |
| Organisation | Spacecraft Checkout Group, UR RAO Satellite Centre, ISRO, Bangalore |
| Supersedes | SCG-GRCD2-SW-2021-01 (SPASDACS 1.0 SRS) |
| Status | Released |

URSC QUALITY POLICY: COMMITTED TO TOTAL QUALITY AND ZERO DEFECT IN SPACE SYSTEMS AND SERVICES

# AUTHENTICATION

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By |  |  |  |
| Reviewed & Approved By |  |  |  |

# CHANGE HISTORY

| Version | Date | Affected Section / Table | Nature [A/M/D] | Description |
| --- | --- | --- | --- | --- |
| 2.0 | March 2026 | All | A – Addition | Initial release of Nova SRS |

* A – Addition    M – Modification    D – Deletion

TABLE OF CONTENTS

# 1.  Introduction

## 1.1  Purpose

This Software Requirements Specification (SRS) defines the detailed software requirements for SPASDACS Nova (version 2.0). SPASDACS Nova is a next-generation, browser-based Spacecraft Status Display and Commanding Software replacing SPASDACS 1.0. The system extends the pictorial block-schematic telemetry display paradigm with a full-stack, microservice backend, real-time telecommanding, tiered user access control, and long-term telemetry storage.

This document translates the functional requirements stated in the SPASDACS Nova Functional Requirements Document (FRD, SCG-GRCD2-SW-2025-02) into precise, verifiable software specifications that shall govern design, implementation, integration, and acceptance testing.

## 1.2  Scope

This SRS covers all software components of the SPASDACS Nova system:

- Frontend SPA (browser-based display, diagram editor, TC panel)

- Backend Go microservices: ingest, gateway, limiter, comparator, storage, simulator, chain-monitor, umacs-tc, umacs-tc-emulator, IAM

- Middleware: Redis (real-time cache), NATS (pub/sub streaming), MongoDB (persistence), InfluxDB (time-series storage)

- Launcher: unified process manager for all backend services

Out of scope: SCC / UMACS hardware, SCOS software, network infrastructure below the application layer.

## 1.3  Definitions, Acronyms and Abbreviations

| Acronym / Term | Definition |
| --- | --- |
| SPASDACS | Spacecraft Status Display and Commanding Software |
| SCC | Spacecraft Checkout Computer |
| PCC | Payload Checkout Computer |
| TM | Telemetry – digitised health and status data from spacecraft |
| TC | Telecommand – command sent to spacecraft or ground checkout instruments |
| SCOS | Special Checkout System |
| SMON | SCOS Monitoring – parameter stream from SCOS data server |
| ADC | Analog-to-Digital Converter monitoring data |
| IAM | Identity and Access Management microservice |
| NATS | Open-source, cloud-native messaging system used for internal pub/sub |
| UDTM | User Display Telemetry Map – user-defined mnemonic-value mapping |
| DTM | Display Telemetry Map Procedure – scripted animation procedure |
| JWT | JSON Web Token – signed access token issued by IAM |
| UMACS | Unified Management and Commanding System |
| RTM | Requirements Traceability Matrix |
| SPA | Single Page Application – browser-based UI served over HTTP |
| DXA | Document eXtension for Authoring – docx unit (1/1440 inch) |
| CHECKNET | Isolated spacecraft checkout LAN |
| IST | Integrated Spacecraft Testing |
| OBT | On-Board Time – spacecraft timestamp |
| FRD | Functional Requirements Document |
| SRS | Software Requirements Specification |

Table 1: Acronyms and Abbreviations

## 1.4  References

| Document Name | Document Number | Date |
| --- | --- | --- |
| SPASDACS Nova Functional Requirements Document | SCG-GRCD2-SW-2025-02 | March 2026 |
| SPASDACS 1.0 Software Requirements Specification | SCG-GRCD2-SW-2021-01 | August 2021 |
| DRC Closeout Minutes – SPASDACS Nova | Internal Meeting Record | March 2026 |
| PASDACS Requirement Document | Not Available | — |

Table 2: References

## 1.5  Overview of the Document

- Section 2 – Overall description, product perspective, and product functions.

- Section 3 – Specific requirements: external interfaces, functional requirements, performance, design constraints, software system attributes, and architecture.

- Section 4 – Requirements Traceability Matrix (SRS-ID → FRD NV-ID → Section → Status).

# 2.  Overall Description

## 2.1  Product Perspective

SPASDACS Nova is a three-tier, browser-based system:

- Tier 1 – Presentation: React single-page application served over HTTP. Runs in any modern Chromium or Firefox browser on the CHECKNET without installation.

- Tier 2 – Application: Go microservices orchestrated by a unified launcher; communicate via REST/WebSocket (external) and NATS pub/sub (internal).

- Tier 3 – Data: MongoDB (diagram persistence, mnemonic catalog, IAM), Redis (real-time telemetry cache, heartbeat), InfluxDB (time-series archive), NATS (streaming).

The system connects to the SCC/PCC WebSocket servers to receive live TM, SMON, and ADC data and forwards those to the browser over NATS-WebSocket bridging. The system can be hosted on the SCC, PCC, or any PC on CHECKNET.

## 2.2  Product Functions

### 2.2.1  Preparatory Requirements

These requirements address diagram authoring, telemetry binding, and user account creation prior to a real-time session:

- Shall provide a browser-based, drag-and-drop diagram editor to create and edit block schematics using shapes (rectangle, circle, ellipse, line, triangle) and imported images.

- Shall support assigning telemetry/SMON/ADC mnemonics to each block, defining which visual attributes (color, border, size, rotation, visibility, position) change based on live values.

- Shall support saving/loading diagrams to/from MongoDB via the Gateway REST API.

- Shall provide IAM-based user management with four built-in roles: super_admin, admin, operator, viewer, plus unauthenticated direct-viewer access.

### 2.2.2  Real-time Display Requirements

These requirements govern the live execution environment where spacecraft data is received and displayed:

- Shall receive live TM, SMON, ADC data from configured SCC/PCC WebSocket servers and publish to browser clients via NATS.

- Shall update visual block attributes in real-time (sub-second latency from SCC data arrival to browser render).

- Shall provide audio and visual alerts on limit violations, data breaks, and invalid/prohibited telemetry combinations.

- Shall support an Auto View mode (slideshow) cycling through selected diagrams with user-configurable dwell time per page.

- Unauthenticated users (direct viewers) SHALL be able to view diagrams and live telemetry in read-only mode without logging in. No telecommanding shall be accessible without authentication.

### 2.2.3  Telecommanding Requirements

Telecommanding is only available to authenticated users with Operator or Admin role:

- Shall provide a TC command panel populated from the TC mnemonic catalog stored in MongoDB.

- Shall integrate with the UMACS TC service to queue and dispatch commands to the spacecraft/instruments via the configured TC endpoint (IP/Port).

- Shall display TC execution status and maintain a historical TC log viewable by authenticated users.

- Shall enforce spacecraft safety by preventing prohibited command combinations as defined in the mnemonic catalog.

### 2.2.4  Identity and Access Management Requirements

The IAM microservice manages authentication and authorisation across the platform:

- Shall support JWT-based stateless authentication with a 15-minute access token and a 7-day refresh token.

- Shall enforce role-based access control (RBAC) via Casbin. TC commands shall be blocked for viewer and unauthenticated users.

- Shall seed default roles (super_admin, admin, operator, viewer) and a default admin user on first startup.

### 2.2.5  Data Management Requirements

- Shall persist mnemonic catalogs (TM and TC) in MongoDB, uploadable via CSV/file through the Gateway API.

- Shall store time-series telemetry in InfluxDB with configurable per-mnemonic storage enable/disable flags.

- Shall maintain UDTM (User Display Telemetry Map) and DTM Procedures in MongoDB with version history.

## 2.3  Constraints

- The system SHALL receive telemetry via WebSocket protocol compliant with the SCC/PCC data publisher interface (see Section 3.1.2).

- The browser client SHALL require no software installation beyond a modern browser (Chromium 110+ or Firefox 110+).

- All telecommand dispatch SHALL transit through the UMACS TC service; direct SCC socket access from the browser is prohibited.

- All backend services SHALL be compiled Go binaries; CGO is disallowed to ensure cross-platform portability.

## 2.4  Assumptions and Dependencies

- TM, SMON, and ADC data publishers are running and accessible on the SCC/PCC at the configured host/port pairs.

- MongoDB, Redis, NATS, and InfluxDB are available on the server node before any SPASDACS Nova service starts.

- The UMACS TC server is running and accessible at the configured TCP endpoint for command dispatch.

- The CHECKNET LAN provides adequate bandwidth for continuous WebSocket streams (minimum 1 Mbps per chain).

- Browser clients have access to the server node on the ports assigned to the gateway (default 8090) and IAM (default 8093) services.

## 2.5  Apportioning of Requirements

The following requirements are planned for future releases and are not part of version 2.0:

- SRS-N-98  Automated audit trail with tamper-evident log storage (MongoDB iam.audit collection).

- SRS-N-99  Diagram Library: shared library of re-usable subsystem block templates across projects.

- SRS-N-100 ACSS (Automated Checkout Sequence System) integration for scripted test execution.

- SRS-N-101 Multi-spacecraft concurrent display on a single SPASDACS Nova instance.

- SRS-N-102 Offline playback mode – replay archived InfluxDB data through the diagram viewer.

# 3.  Specific Requirements

## 3.1  External Interface Requirements

### 3.1.1  Hardware Interfaces

- SCC or PCC running on CHECKNET with Windows or Linux OS, minimum 4 GB RAM.

- Client PCs: any x86-64 host on CHECKNET with a modern browser; minimum 2 GB RAM.

- No dedicated hardware interface card required; all communication is over TCP/IP.

### 3.1.2  TM / SMON / ADC Data Subscription Interface (from SCC WebSocket Server)

SPASDACS Nova connects to the SCC/PCC WebSocket servers as a subscriber client. Each chain (TM1, SMON1, SMON2 …) is configured with a host/port pair in the ingest service YAML configuration.

Connection URL format: ws://{chain.host}:{chain.port}/ws

TM Chain – Subscribe/Unsubscribe (sent by ingest on connect):

{ "action": "subscribe",   "params": ["Frame-id", "OBT"] }

{ "action": "unsubscribe", "params": ["Frame-id", "OBT"] }

TM Data Message format (received per parameter, continuous stream):

{ "param_id":"", "param":"", "source_info":"", "raw_count":"",

"proc_value":"", "time_stamp":"", "upper_limit":"",

"lower_limit":"", "err_desc":"" }

Data break is detected when "break" appears in the err_desc field. On detection, the chain Redis cache is cleared and heartbeat is set to DATA_BREAK.

SCOS/SMON/ADC Chain – Subscribe (connection itself is the subscribe action):

{ "action": "subscribe" }

SCOS Data Message format (received every ~4 s):

{ "paramlist": [{"param":"","value":""},{"param":"","value":""},...],

"stream":"", "seqcount":"", "time":"", "error":"" }

### 3.1.3  Internal NATS Telemetry Streaming Interface

The ingest service publishes normalised telemetry to NATS subjects. The browser frontend subscribes via NATS WebSocket bridge.

### 3.1.4  Gateway REST API Interface

Base URL: http://{server}:8090/api/go/v1

| Method | Endpoint | Description |
| --- | --- | --- |
| POST | /get-telemetry | Fetch current values for a list of mnemonics from Redis |
| GET | /chain-status | Chain heartbeat status (CONNECTED / DATA_BREAK / CONNECTION_FAILED) |
| GET | /chain-mismatches | Comparator chain mismatch list |
| GET | /limit-failures | Active limit failures from limiter service |
| GET | /simulator-status | Simulator running/stopped status |
| GET | /mnemonics/tm | TM mnemonic subsystem list |
| GET | /mnemonics/tm/{subsystem} | TM mnemonics for given subsystem |
| GET | /get/mnemonics/tm | All TM mnemonics (full catalog) |
| GET | /get/mnemonics/tm/{subsystem}/{mn}/range | Limit range for a specific mnemonic |
| GET | /mnemonics/tc | TC mnemonic subsystem list |
| GET | /mnemonics/tc/{subsystem} | TC mnemonics for given subsystem |
| GET | /telecommand/subsystems | List of TC subsystems |
| GET | /telecommand/record | Single TC record by query param |
| GET | /mnemonics/sco | SCO command list |
| GET | /mnemonics/all | All mnemonics (TM + TC + SCO) |
| GET | /telemetry/subsystems | TM subsystem list |
| GET | /tm/mnemonics | Live TM mnemonic list (Redis) |
| POST | /telemetry/upload | Upload TM mnemonic catalog (CSV/JSON) |
| GET | /telemetry/limits/{subsystem} | Fetch editable limits for subsystem |
| PUT | /telemetry/limits | Update single TM mnemonic limits |
| PUT | /telemetry/limits/bulk | Bulk update TM mnemonic limits |
| PUT | /telemetry/tolerance | Update per-mnemonic analog tolerance |
| PUT | /telemetry/expected-value | Update per-mnemonic expected value |
| PUT | /telemetry/ignore-limit-check | Enable/disable limit check per mnemonic |
| PUT | /telemetry/ignore-change-detection | Enable/disable change detection per mnemonic |
| PUT | /telemetry/ignore-chain-comparision | Enable/disable chain comparison per mnemonic |
| PUT | /telemetry/available-chains | Set available chains for a mnemonic |
| POST | /telecommand/upload | Upload TC mnemonic catalog |
| GET | /ud-tm | Fetch current UDTM map |
| POST | /ud-tm | Save new UDTM map version |
| GET | /ud-tm/versions | List all UDTM versions |
| GET | /ud-tm/versions/{version} | Fetch specific UDTM version |
| GET | /dtm/procedures | Fetch current DTM procedures |
| POST | /dtm/procedures | Save DTM procedures |
| GET | /maps/{name} | Read a full Redis hash map as key-value array |
| GET | /diagrams | List all SPASDACS diagrams (meta only) |
| GET | /diagrams/{id} | Fetch a single diagram with full modelData |
| POST | /diagrams | Create or replace a diagram (upsert by id) |
| PATCH | /diagrams/{id} | Patch autoViewInclude / autoViewDuration |
| DELETE | /diagrams/{id} | Delete a diagram |
| PUT | /udtm/values | Update UDTM values in Redis |
| PUT | /dtm/values | Trigger DTM procedure value update |

Table 3: Gateway REST API Endpoints

### 3.1.5  IAM REST API Interface

Base URL: http://{server}:8093/iam

| Method | Endpoint | Auth Required | Description |
| --- | --- | --- | --- |
| POST | /auth/login | None (Public) | Login; returns access_token + refresh_token |
| POST | /auth/refresh | None (Public) | Exchange refresh_token for new access_token |
| POST | /auth/logout | Bearer Token | Revoke refresh token; invalidate session |
| GET | /auth/me | Bearer Token | Return authenticated user profile |
| POST | /auth/change-password | Bearer Token | Change own password; revokes all refresh tokens |
| GET | /roles | Bearer Token | List all roles |
| GET | /roles/{id} | Bearer Token | Get single role by ID |
| POST | /roles | super_admin | Create new role |
| PUT | /roles/{id} | super_admin | Update role description/permissions |
| DELETE | /roles/{id} | super_admin | Delete role |
| GET | /users | iam:users:read | List all users (admin) |
| POST | /users | iam:users:write | Create new user (admin) |
| GET | /users/{id} | Self or admin | Get user profile |
| PUT | /users/{id} | Self or admin | Update user profile / active status |
| DELETE | /users/{id} | iam:users:write | Delete user |
| PUT | /users/{id}/roles | iam:users:write | Assign roles to user |
| GET | /permissions | super_admin | List Casbin permission rules |
| GET | /permissions/resources | super_admin | List all resource identifiers |
| POST | /permissions | super_admin | Add a Casbin policy rule |
| DELETE | /permissions | super_admin | Remove a Casbin policy rule |

Table 4: IAM REST API Endpoints

JWT Token Lifecycle:

### 3.1.6  UMACS Telecommand Interface

The umacs-tc service connects to the UMACS TCP endpoint to dispatch telecommands. Configuration is bootstrapped from YAML and persisted in Redis (hash key: UMACS_ENV_VARIABLES):

| Redis Field | Default | Purpose |
| --- | --- | --- |
| UMACS_TC_IP | — (set in config.yaml) | IP address of UMACS TC server |
| UMACS_TC_PORT | — (set in config.yaml) | TCP port of UMACS TC server |
| UMACS_DATA_SERVER_IP | — (set in config.yaml) | Data server IP for status queries |
| TC_API_REQ_SOURCE | SPASDACS | Source identifier in TC API request |
| TC_API_REQ_PRIORITY | NORMAL | Command priority level |
| TC_API_REQ_EXECUTION_MODE | NORMAL | Execution mode (NORMAL / IMMEDIATE) |
| TC_API_REQ_SUBSYSTEM | ALL | Default subsystem filter |

Table 5: UMACS TC Redis Configuration Fields

### 3.1.7  MongoDB Data Interface

Connection: mongodb://localhost:27017 (configurable). Default database: astra. IAM database: iam.

| Collection | Database | Contents |
| --- | --- | --- |
| spasdacs | astra | Diagram documents – id, name, description, modelData (GoJS JSON), autoViewInclude, autoViewDuration, createdAt, updatedAt |
| tm_mnemonics | astra | TM mnemonic catalog: _id, subsystem, type (BINARY/ANALOG), processingType, range, limits, expectedValue, tolerance, unit, enableComparison, enableLimit, enableStorage |
| tc_mnemonics | astra | TC mnemonic catalog: subsystem, command mnemonics, parameters, safety flags |
| udtm | astra | User Display Telemetry Map versions with timestamp and author |
| dtm_procedures | astra | DTM automation procedures in JSON format |
| users | iam | User accounts: id, username, email, passwordHash (bcrypt), fullName, roles[], isActive, createdAt, updatedAt |
| roles | iam | Role definitions: id, name, description, permissions[], createdAt, updatedAt |
| refresh_tokens | iam | Active refresh tokens: token (hashed), userId, expiresAt (TTL index), createdAt |
| casbin_rules | iam | Casbin RBAC policy rules (ptype, v0–v5) |

Table 6: MongoDB Collections

### 3.1.8  Redis Cache Interface

Connection: localhost:6379 (configurable). Used as the real-time telemetry cache and inter-service shared state store.

| Key Pattern | Description |
| --- | --- |
| TM_{CHAIN}_MAP | Redis hash: param → proc_value for a named chain |
| TM_{CHAIN}_PKT | Redis hash: param → full raw JSON packet for a named chain |
| TM_MAP | Unified Redis hash: all chain params merged (suffix rules applied) |
| HEARTBEAT_{CHAIN} | String: CONNECTED ¦ DATA_BREAK ¦ CONNECTION_FAILED; TTL = 2 s when OK |
| UMACS_ENV_VARIABLES | Hash: UMACS TC configuration fields (see Table 5) |
| LIMIT_FAILURES | Sorted set / hash: currently active limit failures with timestamp |
| CHAIN_MISMATCHES | Hash: comparator mismatches between chain pairs |
| SIMULATOR_STATUS | String: running ¦ stopped |
| UDTM_VALUES | Hash: current UDTM parameter values |

Table 7: Redis Key Patterns

## 3.2  Functional Requirements

### 3.2.1  Diagram Editor (Block Schematic Authoring)

[SRS-N-01 through SRS-N-15 | FRD: NV-01 – NV-22]

- SRS-N-01: The editor SHALL support drawing basic shapes: rectangle, circle, ellipse, line, and triangle on a canvas grid.

- SRS-N-02: The editor SHALL support scaling, rotating, and repositioning shapes with pixel-precision handles.

- SRS-N-03: The editor SHALL provide cut, copy, paste, delete, align, and group operations on shapes and shape groups.

- SRS-N-04: The editor SHALL provide undo/redo stack with a minimum depth of 50 operations.

- SRS-N-05: The editor SHALL allow changing fill color, border color, and border width of each shape.

- SRS-N-06: The editor SHALL allow importing PNG/SVG images as background or custom subsystem block shapes.

- SRS-N-07: The editor SHALL support saving diagrams locally to the browser (JSON file) and remotely to MongoDB via POST /api/go/v1/diagrams.

- SRS-N-08: The editor SHALL support opening diagrams by fetching from GET /api/go/v1/diagrams/{id} and re-hydrating the canvas.

- SRS-N-09: The editor SHALL provide a shape library with user-defined custom subsystem block templates stored per-project.

- SRS-N-10: The editor SHALL support adding descriptive labels and free-text annotations on any canvas element.

- SRS-N-11: The editor SHALL support connector lines/arrows between blocks to indicate signal flow paths (Test path / Signal flow path).

- SRS-N-12: The editor SHALL support conditional visibility rules per block – a block SHALL be hidden or shown based on a configured telemetry value condition.

- SRS-N-13: The editor SHALL provide a multiple-page canvas. Pages SHALL be named and navigable via page-tab strip.

- SRS-N-14: Cross-page navigation links SHALL be configurable on blocks so operators can jump between pages by clicking a block.

- SRS-N-15: The editor SHALL operate in Admin role or higher; Operator, Viewer, and unauthenticated users SHALL have read-only access to diagrams.

### 3.2.2  Telemetry / SMON / ADC Data Binding

[SRS-N-16 through SRS-N-22 | FRD: NV-23 – NV-35]

- SRS-N-16: Each canvas shape SHALL support associating one or more TM/SMON/ADC mnemonics from the MongoDB mnemonic catalog.

- SRS-N-17: The following visual attributes SHALL be programmable per-mnemonic via configurable value-to-attribute mapping rules: fill color, border color, border width, element width, element height, rotation angle, x/y position, and visibility (show/hide).

- SRS-N-18: On mouse-over of any block, a telemetry dropdown SHALL appear displaying the mnemonic name, current processed value, raw count, unit, upper limit, lower limit, and timestamp. Values SHALL update in real-time without closing the dropdown.

- SRS-N-19: The association editor SHALL offer type-ahead autocomplete backed by GET /api/go/v1/mnemonics/tm/{subsystem}, enabling rapid mnemonic selection.

- SRS-N-20: Binding definitions SHALL be persisted as part of the diagram modelData in MongoDB.

- SRS-N-21: The system SHALL support BINARY (digital state) and ANALOG (continuous value) mnemonic types with separate attribute-mapping rule editors.

- SRS-N-22: A signal-flow path highlight mode SHALL allow the user to toggle highlighting of connected blocks and connector lines whose source mnemonic matches a chosen value state.

### 3.2.3  Real-time Viewer

[SRS-N-23 through SRS-N-35 | FRD: NV-36 – NV-56]

- SRS-N-23: The viewer SHALL receive telemetry updates via NATS WebSocket bridge and apply block attribute changes within 500 ms of data arrival at the ingest service.

- SRS-N-24: A TM/SMON/ADC link-status indicator SHALL be displayed in the header for each configured chain. States: CONNECTED (green), DATA_BREAK (amber), CONNECTION_FAILED (red).

- SRS-N-25: On DATA_BREAK, a visual banner alert AND an audio alarm (configurable tone) SHALL be triggered. The banner SHALL remain until data resumes.

- SRS-N-26: When a block transitions to an invalid / prohibited combination, a visual alert (block border flashes red) AND audio alert SHALL activate. The user SHALL be able to define invalid condition logic (multi-mnemonic Boolean expressions).

- SRS-N-27: A state-change blink SHALL accompany any BINARY mnemonic transition. Blink duration SHALL be 1 second; the block shall return to steady state thereafter.

- SRS-N-28: Limit violations (analog value outside lower/upper limits) SHALL cause the associated block fill color to change to the configured alarm color (default: red for high, amber for low) and trigger an audio alert.

- SRS-N-29: Limit violation alerts SHALL remain active until the value returns within limits. A dedicated alarm panel SHALL list all active violations with mnemonic name, value, limit, and timestamp.

- SRS-N-30: The viewer SHALL support zoom in / zoom out (10 %–400 % range) on the canvas, with pan by mouse drag.

- SRS-N-31: A page-selection dropdown SHALL allow switching between all pages of the open diagram.

- SRS-N-32: The viewer SHALL be accessible to unauthenticated users (direct viewer mode) in a read-only capacity. The browser SHALL display live telemetry and diagram state without requiring login. No TC panel, no editor controls SHALL be rendered in unauthenticated mode.

- SRS-N-33: Authenticated Viewer role SHALL have identical read-only access as unauthenticated direct viewer, with additional ability to view TC history log.

- SRS-N-34: The viewer SHALL operate on any PC connected to CHECKNET via a standard browser; no plugins or native installations SHALL be required.

- SRS-N-35: A live value numeric overlay display option SHALL be available per block to show the current processed value as text on the canvas block face.

### 3.2.4  Auto View (Automated Slideshow)

[SRS-N-36 through SRS-N-42 | FRD: NV-57 – NV-63]

- SRS-N-36: The system SHALL provide an Auto View mode that cycles through a user-selected subset of diagrams in sequence.

- SRS-N-37: Each diagram SHALL have an autoViewInclude flag (Boolean) and autoViewDuration (integer, seconds) configurable via PATCH /api/go/v1/diagrams/{id}.

- SRS-N-38: The dwell time per diagram SHALL be configurable from 5 to 3600 seconds.

- SRS-N-39: Auto View SHALL be accessible to all users including unauthenticated direct viewers.

- SRS-N-40: The user SHALL be able to pause, resume, skip-forward, and skip-backward during Auto View.

- SRS-N-41: Any active alarm (data break or limit violation) SHALL cause Auto View to pause on the alarming diagram until the alarm is acknowledged or clears.

- SRS-N-42: The Auto View sequence order SHALL be the order in which diagrams appear in the diagram list (sorted by name ascending by default).

### 3.2.5  Telecommanding

[SRS-N-43 through SRS-N-58 | FRD: NV-64 – NV-76]

- SRS-N-43: Telecommanding SHALL only be accessible to authenticated users with Operator or Admin role. Viewer-role and unauthenticated users SHALL see no TC interface elements.

- SRS-N-44: The TC panel SHALL populate commands from the MongoDB TC mnemonic catalog retrieved via GET /api/go/v1/mnemonics/tc/{subsystem}.

- SRS-N-45: The TC panel SHALL provide subsystem filtering and a search bar for rapid command lookup.

- SRS-N-46: Before dispatching any telecommand, the system SHALL present a confirmation dialog showing command name, parameters, and subsystem to the operator.

- SRS-N-47: Commands SHALL be dispatched to the umacs-tc service, which queues and forwards them to the UMACS TCP endpoint.

- SRS-N-48: The umacs-tc-emulator service SHALL provide a software TC stub for integration testing without a live UMACS connection.

- SRS-N-49: TC execution status (QUEUED, SENT, ACKNOWLEDGED, FAILED) SHALL be shown in real-time in the TC status panel.

- SRS-N-50: The system SHALL maintain a TC history log (list of all dispatched commands with timestamp, user, status) accessible to Operator and Admin roles.

- SRS-N-51: The system SHALL prevent dispatch of prohibited command combinations as defined by safety flags in the TC mnemonic catalog.

- SRS-N-52: Spacecraft safety SHALL be enforced by requiring a two-step confirm (select → confirm → send) with no single-click TC dispatch.

- SRS-N-53: TC commands dispatched from the block-schematic display SHALL be supported (right-click on block → TC shortcut menu) for Admin and Operator roles.

- SRS-N-54: The SCO (Special Checkout Object) command interface SHALL be accessible from the TC panel for instrument-level commands.

- SRS-N-55: TC history log SHALL be persisted in MongoDB and be queryable by subsystem, user, time range.

- SRS-N-56: All TC dispatches SHALL be logged with the authenticated username, timestamp, command, and execution result for audit purposes.

- SRS-N-57: The TC API request parameters (source, priority, execution mode, subsystem) SHALL be configurable at runtime via Redis (UMACS_ENV_VARIABLES) without service restart.

- SRS-N-58: The system SHALL display the current TC configuration (IP, port, source, priority, mode) in the TC settings panel accessible to Admin role.

### 3.2.6  Data Persistence – Diagram CRUD

[SRS-N-59 through SRS-N-65 | FRD: NV-77 – NV-85]

- SRS-N-59: Diagrams SHALL be persisted in MongoDB spasdacs collection as documents containing id (string UUID), name, description, modelData (GoJS JSON), backgroundColor, autoViewInclude, autoViewDuration, createdAt, updatedAt.

- SRS-N-60: The POST /diagrams endpoint SHALL perform an upsert (ReplaceOne with upsert=true) keyed on the diagram id field, preserving createdAt on update.

- SRS-N-61: The DELETE /diagrams/{id} endpoint SHALL return HTTP 404 if the diagram does not exist and HTTP 200 on success.

- SRS-N-62: The list endpoint (GET /diagrams) SHALL return SpasdacsMeta objects only (no modelData) to minimise payload size for the diagram selector UI.

- SRS-N-63: The full diagram including modelData SHALL only be returned by GET /diagrams/{id}.

- SRS-N-64: Diagram names SHALL be unique per deployment. A uniqueness check SHALL be enforced at the API layer before persisting.

- SRS-N-65: Diagrams SHALL be exportable as JSON files from the browser for backup or transfer between deployments.

### 3.2.7  Mnemonic Catalog Management

[SRS-N-66 through SRS-N-72 | FRD: NV-86 – NV-93]

- SRS-N-66: TM mnemonic catalog SHALL be uploadable via POST /api/go/v1/telemetry/upload accepting CSV or JSON file format.

- SRS-N-67: Each TM mnemonic document SHALL carry fields: _id, subsystem, type (BINARY/ANALOG), processingType, range, limits, expectedValue, tolerance, unit, digitalStatus, cdbMnemonic, sourceFile, enableComparison, enableLimit, enableStorage.

- SRS-N-68: The limits, tolerance, expectedValue, enableComparison, enableLimit, enableStorage fields SHALL be individually updatable at runtime without re-uploading the catalog (PUT /telemetry/limits, /tolerance, /expected-value, /ignore-* endpoints).

- SRS-N-69: TC mnemonic catalog SHALL be uploadable via POST /api/go/v1/telecommand/upload.

- SRS-N-70: The mnemonic catalog SHALL support per-subsystem querying for Monaco editor autocomplete.

- SRS-N-71: The system SHALL maintain backward compatibility if a mnemonic tolerance value is stored as a string (FlexFloat64 decoder).

- SRS-N-72: Bulk limit update SHALL be supported via PUT /telemetry/limits/bulk accepting an array of {mnemonic, upper, lower} objects.

### 3.2.8  UDTM / DTM Procedure Management

[SRS-N-73 through SRS-N-78 | FRD: NV-94 – NV-99]

- SRS-N-73: UDTM (User Display Telemetry Map) defines a user-configurable mapping of mnemonic names to display values. The current version SHALL be retrievable via GET /ud-tm.

- SRS-N-74: A new UDTM version SHALL be created by POST /ud-tm. All previous versions SHALL remain accessible via GET /ud-tm/versions/{version} for rollback.

- SRS-N-75: DTM Procedures define scripted animation sequences. They SHALL be stored in MongoDB via POST /dtm/procedures and applied to Redis at runtime via PUT /dtm/values.

- SRS-N-76: DTM procedure updates SHALL publish a NATS message on subject DTM_PROCEDURES_UPDATED to notify subscribed frontend clients.

- SRS-N-77: UDTM and DTM changes SHALL only be performable by users with Admin role or higher.

- SRS-N-78: The current UDTM hash-map values SHALL be readable from Redis via GET /maps/{name} endpoint for debugging and verification.

### 3.2.9  Identity and Access Management (IAM)

[SRS-N-79 through SRS-N-95 | FRD: NV-100 – NV-116]

- SRS-N-79: The IAM microservice SHALL run as an independent Go service on port 8093 (configurable).

- SRS-N-80: The IAM service SHALL use MongoDB database iam with collections users, roles, refresh_tokens, casbin_rules.

- SRS-N-81: On first startup, the IAM service SHALL seed four default roles (super_admin, admin, operator, viewer) and a default admin user (username: admin, password: Admin@123!) if the database is empty.

- SRS-N-82: Passwords SHALL be stored as bcrypt hashes (minimum cost factor 12). Plain-text passwords SHALL never be stored or logged.

- SRS-N-83: Access tokens SHALL be JWT signed with HMAC-SHA256 using a configurable secret key. Token TTL SHALL be 15 minutes.

- SRS-N-84: Refresh tokens SHALL be opaque random strings stored as SHA-256 hashes in MongoDB with a TTL index of 7 days.

- SRS-N-85: The IAM service SHALL use Casbin with MongoDB adapter for RBAC policy enforcement. Policy rules SHALL persist across restarts.

- SRS-N-86: The four built-in roles SHALL have the following default permissions:

  - super_admin: iam:roles:*, iam:users:*, iam:permissions:*

  - admin: * (full access to all resources)

  - operator: telemetry:read, telemetry:write, commands:read, commands:write, simulator:read, simulator:write

  - viewer: telemetry:read, commands:read, simulator:read

- SRS-N-87: Unauthenticated (direct viewer) access SHALL be permitted for diagram view and live telemetry display only. No TC, no editor, no user management SHALL be accessible without a valid token.

- SRS-N-88: Role creation, editing, and deletion SHALL only be performed by super_admin.

- SRS-N-89: User creation, deletion, and role assignment SHALL require iam:users:write permission.

- SRS-N-90: A user SHALL be able to read and update their own profile (GET/PUT /users/{id}) without admin permission (self-service).

- SRS-N-91: The system SHALL support account locking via isActive=false. Locked accounts SHALL be denied token issuance.

- SRS-N-92: The /iam/auth/me endpoint SHALL return the full user profile including assigned roles for the currently authenticated user.

- SRS-N-93: The IAM service SHALL expose a health check at GET /health returning { "status":"ok", "service":"iam" } without authentication.

- SRS-N-94: All IAM endpoints SHALL be served over HTTP on the internal CHECKNET; TLS is deferred to future network layer (not a v2.0 requirement).

- SRS-N-95: The Gateway service SHALL forward JWT validation errors as HTTP 401 Unauthorized to API clients.

### 3.2.10  Limit Monitoring and Alerting

[SRS-N-96 – SRS-N-97 | FRD: NV-117 – NV-120]

- SRS-N-96: The limiter service SHALL continuously compare incoming TM values against the upper/lower limits stored in the mnemonic catalog. Violations SHALL be written to Redis LIMIT_FAILURES key.

- SRS-N-97: ANALOG mnemonics SHALL support a configurable tolerance value to suppress spurious limit failure alerts on noisy channels.

### 3.2.11  Chain Comparator

[SRS-N-98 | FRD: NV-121]

- SRS-N-98: The comparator service SHALL compare values between parallel chains (e.g., TM1 vs TM2). Mismatches SHALL be written to Redis CHAIN_MISMATCHES key and exposed via GET /chain-mismatches. Per-mnemonic comparison can be disabled via enableComparison flag.

### 3.2.12  Chain Monitor / Health Watchdog

- SRS-N-99: The chainmon service SHALL periodically poll each chain heartbeat key in Redis and publish chain health status to NATS for consumption by the frontend chain-status indicator.

### 3.2.13  Telemetry Storage (InfluxDB)

- SRS-N-100: The storage service SHALL write telemetry values to InfluxDB for mnemonics with enableStorage=true. The InfluxDB database, org, and token SHALL be configurable.

### 3.2.14  Simulator

[SRS-N-101 – SRS-N-102 | FRD: NV-122 – NV-123]

- SRS-N-101: The simulator service SHALL generate synthetic TM/SMON/ADC data conforming to the mnemonic catalog ranges, allowing diagram testing without a live SCC connection.

- SRS-N-102: Simulator start/stop SHALL be controllable from the UI. Status SHALL be readable via GET /simulator-status.

## 3.3  Performance Requirements

- The Gateway service SHALL handle a minimum of 100 concurrent WebSocket/REST connections without degradation.

- End-to-end latency from SCC WebSocket data arrival to browser render SHALL not exceed 500 ms under nominal load.

- The ingest service SHALL process a minimum of 500 TM parameter updates per second per chain with no Redis pipeline failures.

- MongoDB diagram read (GET /diagrams/{id}) SHALL respond within 100 ms for diagrams up to 2 MB modelData.

- The system SHALL operate on server hardware with minimum 4 GB RAM and a 4-core 2 GHz processor.

- Browser clients SHALL require minimum 2 GB RAM; the UI SHALL be responsive on a laptop-class machine.

- The IAM service SHALL issue a JWT access token within 200 ms of a valid login request under nominal load.

## 3.4  Design Constraints

- All backend services SHALL be written in Go with CGO disabled (CGO_ENABLED=0) to produce statically linked, platform-portable binaries.

- The browser UI SHALL be a React single-page application served as static assets by the Gateway service or a dedicated file server; no server-side rendering.

- TM/SMON/ADC data interfaces SHALL be WebSocket-based, matching the existing SCC data publisher protocol defined in Section 3.1.2.

- All inter-service messaging SHALL use NATS pub/sub; direct service-to-service HTTP calls are prohibited except for health checks.

- The system SHALL run on Windows 10+ and Ubuntu 20.04+ without modification to the compiled binaries.

- MongoDB, Redis, NATS, and InfluxDB are external dependencies; the system does not bundle or manage these services.

- The system SHALL use chi v5 as the HTTP router. No other HTTP frameworks are permitted.

- Configuration SHALL use YAML files managed by Viper. Environment variable overrides SHALL be supported.

## 3.5  Software System Attributes

| Attribute | Requirement |
| --- | --- |
| Reliability | All backend services SHALL implement exponential back-off reconnect for WebSocket and NATS connections. The ingest service SHALL log and resume automatically after a chain disconnect within 30 seconds. |
| Availability | The system SHALL support hot-restart of individual microservices without data loss from MongoDB or InfluxDB. Redis cache may be cleared on restart; diagrams are always reloaded from MongoDB. |
| Maintainability | Each microservice SHALL be independently deployable. Services SHALL communicate only via NATS and shared Redis/MongoDB; no hard-coded cross-service HTTP calls. Configuration is YAML-driven with Viper defaults. |
| Security | All TC dispatches require authenticated Operator/Admin role with JWT. Passwords stored as bcrypt hashes. Refresh tokens stored as SHA-256 hashes with TTL. Casbin enforces RBAC for all IAM-protected endpoints. |
| Portability | Statically compiled Go binaries run on Windows 10+ and Ubuntu 20.04+ x86-64. No runtime dependencies beyond the external middleware stack. |
| Usability | Displays SHALL be aesthetically pleasing and allow at-a-glance detection of spacecraft state anomalies. All interactive controls SHALL be accessible within two mouse clicks from the main display. |
| Traceability | All TC commands SHALL be logged with username, timestamp, command, and outcome. All IAM changes SHALL generate a log entry. |
| Extensibility | New microservices SHALL follow the cmd/internal/service Go package layout and register with the unified Launcher. New telemetry chains are added via YAML configuration only. |

Table 8: Software System Attributes

## 3.6  Software Architecture Overview

| Service | Port | Responsibility |
| --- | --- | --- |
| launcher | — | Unified process manager; starts/monitors all services with configurable delays and restart policies |
| ingest | — | WebSocket subscriber for TM, SMON, ADC chains; writes to Redis; publishes to NATS |
| gateway | 8090 | REST + WebSocket API gateway; serves diagram CRUD, mnemonic catalog, telemetry queries, TC upload; bridges NATS to browser |
| limiter | — | Watches Redis TM_MAP; compares against catalog limits; writes LIMIT_FAILURES to Redis |
| comparator | — | Cross-chain value comparison; writes CHAIN_MISMATCHES to Redis |
| chainmon | — | Polls chain heartbeats; publishes health status to NATS |
| storage | — | Reads Redis TM_MAP; writes time-series data to InfluxDB for enabled mnemonics |
| simulator | — | Generates synthetic TM/SMON data into Redis for offline diagram testing |
| umacs-tc | — | REST endpoint receiving TC dispatch requests from gateway; forwards to UMACS TCP socket |
| umacs-tc-emulator | — | Software stub for umacs-tc; used during integration testing without live UMACS |
| iam | 8093 | Identity & Access Management; JWT issuance, RBAC via Casbin, user/role CRUD against MongoDB iam database |

Table 9: Microservice Architecture Summary

# 4.  Requirements Traceability Matrix

The following table maps each SRS requirement to its source FRD requirement, document section, and implementation status.

| SRS ID | Section | FRD Req ID | Description | Status |
| --- | --- | --- | --- | --- |
| SRS-N-01 | 3.2.1 | NV-01 | Diagram editor – basic shape drawing | Implemented |
| SRS-N-02 | 3.2.1 | NV-02 | Diagram editor – scale, rotate, reposition shapes | Implemented |
| SRS-N-03 | 3.2.1 | NV-03 | Diagram editor – cut / copy / paste / delete / align / group | Implemented |
| SRS-N-04 | 3.2.1 | NV-04 | Diagram editor – undo/redo (depth ≥ 50) | Implemented |
| SRS-N-05 | 3.2.1 | NV-05 | Diagram editor – fill / border color / border width | Implemented |
| SRS-N-06 | 3.2.1 | NV-06 | Diagram editor – import PNG/SVG images | Implemented |
| SRS-N-07 | 3.2.1 | NV-07 | Diagram save to MongoDB via POST /diagrams | Implemented |
| SRS-N-08 | 3.2.1 | NV-08 | Diagram open from MongoDB via GET /diagrams/{id} | Implemented |
| SRS-N-09 | 3.2.1 | NV-09 | Custom shape library per project | Implemented |
| SRS-N-10 | 3.2.1 | NV-10 | Labels and text annotations on canvas | Implemented |
| SRS-N-11 | 3.2.1 | NV-11 | Connector lines / signal-flow arrows | Implemented |
| SRS-N-12 | 3.2.1 | NV-12 | Conditional block visibility (hide/show per TM value) | Implemented |
| SRS-N-13 | 3.2.1 | NV-13 | Multi-page canvas with named page tabs | Implemented |
| SRS-N-14 | 3.2.1 | NV-14 | Cross-page navigation links on blocks | Implemented |
| SRS-N-15 | 3.2.1 | NV-15 | Editor access restricted to Admin+ role | Implemented |
| SRS-N-16 | 3.2.2 | NV-23 | Associate TM/SMON/ADC mnemonics to shapes | Implemented |
| SRS-N-17 | 3.2.2 | NV-24 | Programmable visual attributes (color, size, rotate, pos, hide) | Implemented |
| SRS-N-18 | 3.2.2 | NV-25 | Mouse-over TM dropdown with real-time values | Implemented |
| SRS-N-19 | 3.2.2 | NV-26 | Monaco autocomplete from mnemonic catalog | Implemented |
| SRS-N-20 | 3.2.2 | NV-27 | Binding persistence in diagram modelData | Implemented |
| SRS-N-21 | 3.2.2 | NV-28 | BINARY and ANALOG mnemonic binding support | Implemented |
| SRS-N-22 | 3.2.2 | NV-29 | Signal-flow path highlight mode | Implemented |
| SRS-N-23 | 3.2.3 | NV-36 | Real-time TM update latency ≤ 500 ms | Implemented |
| SRS-N-24 | 3.2.3 | NV-37 | Chain link-status indicator (CONNECTED/DATA_BREAK/FAILED) | Implemented |
| SRS-N-25 | 3.2.3 | NV-38 | Data-break visual banner and audio alarm | Implemented |
| SRS-N-26 | 3.2.3 | NV-39 | Invalid/prohibited combination visual+audio alert | Implemented |
| SRS-N-27 | 3.2.3 | NV-40 | BINARY state-change blink (1 s duration) | Implemented |
| SRS-N-28 | 3.2.3 | NV-41 | Limit violation block color change and audio alert | Implemented |
| SRS-N-29 | 3.2.3 | NV-42 | Persistent alarm panel with mnemonic / value / limit / time | Implemented |
| SRS-N-30 | 3.2.3 | NV-43 | Canvas zoom 10%–400% and pan | Implemented |
| SRS-N-31 | 3.2.3 | NV-44 | Page-selection dropdown | Implemented |
| SRS-N-32 | 3.2.3 | NV-45 | Unauthenticated direct viewer (read-only, no TC) | Implemented |
| SRS-N-33 | 3.2.3 | NV-46 | Viewer role – read-only + TC history log | Implemented |
| SRS-N-34 | 3.2.3 | NV-47 | Browser-only access, no client installation | Implemented |
| SRS-N-35 | 3.2.3 | NV-48 | Live numeric value overlay on block face | Implemented |
| SRS-N-36 | 3.2.4 | NV-57 | Auto View slideshow mode | Implemented |
| SRS-N-37 | 3.2.4 | NV-58 | Per-diagram autoViewInclude and autoViewDuration flags | Implemented |
| SRS-N-38 | 3.2.4 | NV-59 | Configurable dwell time 5–3600 s per diagram | Implemented |
| SRS-N-39 | 3.2.4 | NV-60 | Auto View accessible to all users including unauthenticated | Implemented |
| SRS-N-40 | 3.2.4 | NV-61 | Pause / resume / skip-forward / skip-backward controls | Implemented |
| SRS-N-41 | 3.2.4 | NV-62 | Auto View pauses on alarming diagram until cleared | Implemented |
| SRS-N-42 | 3.2.4 | NV-63 | Sequence order by diagram name ascending | Implemented |
| SRS-N-43 | 3.2.5 | NV-64 | TC access restricted to Operator+ role | Implemented |
| SRS-N-44 | 3.2.5 | NV-65 | TC panel populated from MongoDB TC mnemonic catalog | Implemented |
| SRS-N-45 | 3.2.5 | NV-66 | TC subsystem filter and command search bar | Implemented |
| SRS-N-46 | 3.2.5 | NV-67 | TC confirmation dialog before dispatch | Implemented |
| SRS-N-47 | 3.2.5 | NV-68 | Commands dispatched via umacs-tc service to UMACS TCP | Implemented |
| SRS-N-48 | 3.2.5 | NV-69 | umacs-tc-emulator for integration testing | Implemented |
| SRS-N-49 | 3.2.5 | NV-70 | Real-time TC execution status (QUEUED/SENT/ACK/FAILED) | Implemented |
| SRS-N-50 | 3.2.5 | NV-71 | TC history log (Operator+ access) | Implemented |
| SRS-N-51 | 3.2.5 | NV-72 | Prohibited command combination prevention | Implemented |
| SRS-N-52 | 3.2.5 | NV-73 | Two-step confirm for spacecraft safety (select→confirm→send) | Implemented |
| SRS-N-53 | 3.2.5 | NV-74 | Block right-click TC shortcut menu (Admin/Operator) | Implemented |
| SRS-N-54 | 3.2.5 | NV-75 | SCO command interface accessible from TC panel | Implemented |
| SRS-N-55 | 3.2.5 | NV-76 | TC history persisted in MongoDB, queryable | Implemented |
| SRS-N-56 | 3.2.5 | NV-76 | TC audit log: username, timestamp, command, result | Implemented |
| SRS-N-57 | 3.2.5 | NV-76 | TC API params runtime-configurable via Redis (no restart) | Implemented |
| SRS-N-58 | 3.2.5 | NV-76 | TC config panel (Admin): shows IP/port/source/priority/mode | Implemented |
| SRS-N-59 | 3.2.6 | NV-77 | Diagram MongoDB document schema (id, name, modelData, …) | Implemented |
| SRS-N-60 | 3.2.6 | NV-78 | POST /diagrams – upsert preserving createdAt | Implemented |
| SRS-N-61 | 3.2.6 | NV-79 | DELETE /diagrams/{id} – 404 on missing | Implemented |
| SRS-N-62 | 3.2.6 | NV-80 | GET /diagrams returns metadata only (no modelData) | Implemented |
| SRS-N-63 | 3.2.6 | NV-81 | GET /diagrams/{id} returns full modelData | Implemented |
| SRS-N-64 | 3.2.6 | NV-82 | Diagram name uniqueness enforced at API layer | Implemented |
| SRS-N-65 | 3.2.6 | NV-83 | Diagram JSON export from browser for backup/transfer | Implemented |
| SRS-N-66 | 3.2.7 | NV-86 | TM catalog upload via POST /telemetry/upload (CSV/JSON) | Implemented |
| SRS-N-67 | 3.2.7 | NV-87 | TM mnemonic document schema (all fields) | Implemented |
| SRS-N-68 | 3.2.7 | NV-88 | Runtime editable: limits, tolerance, expectedValue, flags | Implemented |
| SRS-N-69 | 3.2.7 | NV-89 | TC catalog upload via POST /telecommand/upload | Implemented |
| SRS-N-70 | 3.2.7 | NV-90 | Per-subsystem mnemonic query for Monaco autocomplete | Implemented |
| SRS-N-71 | 3.2.7 | NV-91 | FlexFloat64 tolerance backward compatibility (string→float) | Implemented |
| SRS-N-72 | 3.2.7 | NV-92 | Bulk limit update via PUT /telemetry/limits/bulk | Implemented |
| SRS-N-73 | 3.2.8 | NV-94 | UDTM current version via GET /ud-tm | Implemented |
| SRS-N-74 | 3.2.8 | NV-95 | UDTM versioning – POST creates new, GET /{version} retrieves old | Implemented |
| SRS-N-75 | 3.2.8 | NV-96 | DTM procedures – save/apply; publishes DTM_PROCEDURES_UPDATED | Implemented |
| SRS-N-76 | 3.2.8 | NV-97 | NATS notification on DTM procedure update | Implemented |
| SRS-N-77 | 3.2.8 | NV-98 | UDTM/DTM changes restricted to Admin+ role | Implemented |
| SRS-N-78 | 3.2.8 | NV-99 | UDTM Redis map readable via GET /maps/{name} | Implemented |
| SRS-N-79 | 3.2.9 | NV-100 | IAM independent microservice on port 8093 | Implemented |
| SRS-N-80 | 3.2.9 | NV-101 | IAM MongoDB database iam (users, roles, tokens, casbin) | Implemented |
| SRS-N-81 | 3.2.9 | NV-102 | First-startup seed: default roles + admin user | Implemented |
| SRS-N-82 | 3.2.9 | NV-103 | Passwords stored as bcrypt hashes (cost ≥ 12) | Implemented |
| SRS-N-83 | 3.2.9 | NV-104 | JWT access token HMAC-SHA256, TTL = 15 min | Implemented |
| SRS-N-84 | 3.2.9 | NV-105 | Refresh token – opaque hash, TTL = 7 days, MongoDB TTL index | Implemented |
| SRS-N-85 | 3.2.9 | NV-106 | Casbin RBAC with MongoDB adapter | Implemented |
| SRS-N-86 | 3.2.9 | NV-107 | Default role permissions (super_admin/admin/operator/viewer) | Implemented |
| SRS-N-87 | 3.2.9 | NV-108 | Unauthenticated direct viewer – no TC, no editor | Implemented |
| SRS-N-88 | 3.2.9 | NV-109 | Role CRUD restricted to super_admin | Implemented |
| SRS-N-89 | 3.2.9 | NV-110 | User CRUD requires iam:users:write | Implemented |
| SRS-N-90 | 3.2.9 | NV-111 | Self-service: user can read/update own profile | Implemented |
| SRS-N-91 | 3.2.9 | NV-112 | Account locking via isActive=false | Implemented |
| SRS-N-92 | 3.2.9 | NV-113 | /iam/auth/me returns full profile + roles | Implemented |
| SRS-N-93 | 3.2.9 | NV-114 | IAM health check at GET /health (no auth) | Implemented |
| SRS-N-94 | 3.2.9 | NV-115 | HTTP on CHECKNET; TLS deferred | Implemented |
| SRS-N-95 | 3.2.9 | NV-116 | Gateway JWT validation → HTTP 401 on failure | Implemented |
| SRS-N-96 | 3.2.10 | NV-117 | Limiter: limit violations written to Redis LIMIT_FAILURES | Implemented |
| SRS-N-97 | 3.2.10 | NV-118 | Limiter: per-mnemonic analog tolerance for noise suppression | Implemented |
| SRS-N-98 | 3.2.11 | NV-121 | Comparator: cross-chain mismatch detection + Redis storage | Implemented |
| SRS-N-99 | 3.2.12 | NV-119 | Chainmon: heartbeat polling + NATS health status publish | Implemented |
| SRS-N-100 | 3.2.13 | NV-120 | Storage: InfluxDB time-series write for enabled mnemonics | Implemented |
| SRS-N-101 | 3.2.14 | NV-122 | Simulator: synthetic TM data generation from catalog ranges | Implemented |
| SRS-N-102 | 3.2.14 | NV-123 | Simulator: start/stop from UI via GET /simulator-status | Implemented |
| SRS-N-98F | 2.5 | — | Audit trail (tamper-evident log in iam.audit) | Deferred |
| SRS-N-99F | 2.5 | — | Diagram block template library | Deferred |
| SRS-N-100F | 2.5 | — | ACSS automated checkout sequence integration | Deferred |
| SRS-N-101F | 2.5 | — | Multi-spacecraft concurrent display | Deferred |
| SRS-N-102F | 2.5 | — | Offline InfluxDB playback mode | Deferred |

Table 10: Requirements Traceability Matrix

| Key | Meaning |
| --- | --- |
| A | Addition (new requirement) |
| M | Modification of existing requirement |
| D | Deletion of previous requirement |