SOFTWARE REQUIREMENTS SPECIFICATION

Telecommand Software 2.1

GEOSAT RF & Payload Checkout Division-2

Spacecraft Checkout Group

JULY 2020

UR RAO SATELLITE CENTRE

INDIAN SPACE RESEARCH ORGANISATION

BANGALORE

AUTHENTICATION

|  | Name | Signature | Date |
| --- | --- | --- | --- |
| Prepared By | Janardhana Reddy G |  |  |
| Reviewed By | -- |  |  |
| Approved By | M.S. Bhuvaneswari |  |  |

CHANGE HISTORY

| Version Number | Date | Affected Section, Figure, Table | Nature of change [A,M,D] * | Descriptiosssn |
| --- | --- | --- | --- | --- |
| 2.0 | 1-11-2017 | - | New |  |
| 2.1 | 01-07-2020 |  | A | New Feature Requirements added |

* A – Addition, D – Deletion, M-Modification

TABLE OF CONTENTS

Contents

1.	Introduction	7

1.1 Purpose	7

1.2 Scope	8

1.3Definitions, Acronyms and Abbreviations	8

1.3.1 Definitions	8

1.3.2 Acronyms &Abbreviations	8

1.4 References	9

1.5 Overview of the document	9

2.	Overall Description	10

2.1 Product Perspective	10

2.2 Product Functions	10

2.3 Constraints	11

2.4 Assumptions and Dependencies	11

2.5 Apportioning of requirements	12

3.	Specific Requirements	12

3.1 External Interface Requirements	12

3.1.1 Spacecraft Checkout Computer	12

3.1.2 User Interfaces	12

3.1.3 Software Interfaces	12

3.2 Functional Requirements	13

3.2.1	Database tables for Telecommands	13

3.2.2 Provision to send tele-commands	15

3.2.3 Provision to trigger file execution.	15

3.2.4 Provisionto send configuration based BOA.	16

3.2.5 Provision to send required BOA manually.	16

3.2.6 Provision to send configuration based commands.	17

3.2.7 Search option to select commands or files	17

3.2.8 Provision should be there for command grouping	17

3.2.9Application should be accessible from any PC in the network	17

3.2.10 Safe re-configuration of payload from any current state to any another state shall be available. On/off sequencing shall be taken care.	17

3.2.11GUI interface shall be there to show current state of payload with switching complexity to change from current payload configurations to new configurations.	17

3.2.12Provision shall be there to generate test procedures document.	18

3.2.13Provision shall be there to exclude on commands while configuring the payload.	18

3.2.14Provision shall be there to set configuration based on test parameter.	18

3.2.15Provision shall be there to include call files along with other commands while setting configuration.	18

3.2.16Provision shall be there to inhibit tele-commands.	18

3.2.17Provision shall be there to limit the number of TWTAs /SSPAs to be switched on based on current limit and/or thermal requirement.	18

3.2.18Provision shall be there to keep adjacent channels at maximum BOA.	18

3.2.19 Provision to generate incremental procedure files	19

3.3. Performance Requirements	40

3.4. Design Constraints	40

3.5. Software System attributes	40

4.	Requirements Traceability Matrix	42

5.	Comparison of Featuresof all versions of TelecommandApplication	44

List of Tables

Table 1 Acronyms & Abbreviations	8

Table 2 References	8

Table 3 Telecommand Database Table	12

Table 4 Configuration based BOA database Table	12

Table 5 Telecommand files database table	12

Table 6 Data commands Database	13

Table 7 Data Commands Mapping Table	13

Table 8 Configuration based commanding database	14

Table 9 SCC Details	14

Table 10 Requirements Traceability	20

Table 11 Features Comparison	22

List of Figures

Figure 1 Telecommand software Package Interfaces	9

Figure 2 GUI for Tele-commands Drop down	14

Figure 3 GUI for selecting Tele-command Files	15

Figure 4  GUI for selecting BOA commands	15

Figure 5 GUI for Sending Data commands	15

Figure 6 GUI for sending configuration based commands	16

# Introduction

1.1 Purpose

1.1.1 Brief overview of the existing Telecommanding Software ver1.0 & ver2.0

Prior to the characterization of Communication Payloads,they have to be commanded for setting up the required configuration. “Telecommanding Utilities” are one of the associated packages of COMPACSS meant for generation of payload telecommands based on users’ requirement & formatting them into a CCL-compatible file and transferring the same to SCC via ftp over LAN with UDP protocol with a request for execution. Following modes of  telecommand(s) selection are supported.

Mode-1:  Individual telecommand(s)

In this mode, user can select individual telecommand(s) from a drop-down list box and   transmit them to SCC. The selected telecommands are formatted into a CCL-compatible file and transferred to SCC with a request for execution using an existing protocol               between PCC &SCC. The telecommands are available in the database in MS-Access.

Mode-2:  SCC-resident telecommand sequence file(s)

In this mode, the telecommand sequence files are already available in the SCC and the list of those filenames are available in the database in PCC. User can specify the file to be executed after which the PCC places a request with SCC for execution.

Mode-3:  Configuration number(s)

In this mode, user selects only the configuration no(s) while the software picks up the corresponding telecommand(s) & telemetries from the database and formats them into CCL-compatible file and transfers it to SCC with a request for execution

This has greatly helped the test engineer to dynamically generate the command sequence files for the required configuration(s) on the Payload thereby reducing the preparation time as well as the execution time while avoiding manual errors. This was extensively used in all GEOSAT payload checkouts from INSAT-3A onwards upto GSAT-19. This was developed in Borland C++ 5.0 with WinAPI under Windows. The obsolescence of the development platform forced migration of the software into a new Telecommanding software 2.0 developed with python and vue XXXX  vide SRS doc.

This version (ver 2.0) fulfilled some of the following limitations of ver 1.0

- When the total number of tele-commands is huge it is difficult & time-consuming to navigate through the list box to search & pick the required command.

- SEND TC is not preceded by the corresponding EXPECTED Tm in Mode-1.This limitation was not there in other Modes.

- Configuration will be set using incremental commanding, with this software will not check any constrains before turning on the payload. User must ensure sequence of configuration selection.

Additionally the following requirements were also fulfilled by ver. 2.0

- The package was developed as a web-application to serve the purpose of ‘remote-commanding’ in future during checkout operations.

Even with all these features, it was proposed to introduce smart features and artificial intelligence-based commanding to achieve complete automation of payload reconfiguration and to support optimal switching sequence. Telecommanding ver 2.1 is proposed to be developed to fulfill these requirements.

- Safe re-configuration of payload from any current state to any another state

- Optimal payload test configuration sequence generation in runtime based on current state of payload.

- Sending data commands in manual mode.

- Additional features to support GISAT Series of satellites.

All the features of this software are tabulated in features Comparison table in section 5 of this document.This shall be called Telecommand package ver 2.1 or  “SMART-TC” and shall co-exist with other packages of COMPACSS/NAVPACSS/METPACSS and GIMPACSS.

1.2 Scope

This document outlines the requirements of the updated version of the Telecommand software product ‘SMART-TC’ which is proposed to be developed. This document is an outcome of requirements generated by existing software, experience gained during the RF Testing of various satellites and after discussions and feedback from the user teams.

1.3Definitions, Acronyms and Abbreviations

### 1.3.1 Definitions

ODBC: Open Database Connectivity. ODBC is a programming interface that enables applications to access data in database management systems that uses Structured Query Language (SQL) as a data access standard.

### 1.3.2 Acronyms &Abbreviations

| S. No | Acronym / Abbreviation | Definition |
| --- | --- | --- |
|  | COMPACSS | Communication Payload Checkout System Software |
|  | SCC | Spacecraft Checkout Computer |
|  | TM | Telemetry |
|  | GUI | Graphical User Interface |
|  | ODBC | Open Database Connectivity |
|  | GISAT | GEO Imaging Satellite |
|  | NAVPACSS | Navigation Payload Checkout System Software |
|  | METPACSS | Metrological Payload Checkout System Software |
|  | GIMPACSS | Geo Imaging Payload Checkout System Software |
|  | BAS | Battery Simulator |
|  | TM | Telemetry |
|  | TC | Telecommand |

Table 1 Acronyms & Abbreviations

1.4 References

| Document Name | Document Number | Publishing Date |
| --- | --- | --- |

Table 2References

1.5 Overview of the document

This document describes the functional and interface requirements of the Telecommand Software version 2.1

Section 2 gives the overall description of the Software

Section 3 gives the detailed description of functional and interface requirements

Section 4 lists the requirement traceability matrix

Section 5 lists the feature comparison.

# Overall Description

Telecommand software product will be used to send tele-commands to SCC either in stand-alone mode or through COMPACSS from payload Checkout Computer.

This application shall be used during checkout operations of GEOSAT satellites. The Utility uses data driven approach so that it shall be easily re-configured for varying test configuration.

2.1 Product Perspective

The application shall be implemented on a PC with windows operating System. It shall interface to the Spacecraft checkout computer via LAN with UDP &FTP protocol.

Telecommand

Software

COMPACS

Spacecraft Checkout Computer

GUI(Web)

TMTC Database

Figure 1Telecommand software Package Interfaces

2.2 Product Functions

This software uses a data driven approach, which means the software, is flexible enough to be configured for different payload architectures.

The major functional requirements of the software are:

- Provision to send tele-commands by selecting from a dropdown list.

- Provision to trigger file execution for those files which are already stored in SCC.

- Provision to send configuration based BOA commands.

- Provision to send required BOA data command manually.

- Provision to send configuration based commands.

- Search option to select commands or files.

- Commands grouping option shall be available.

- Application shall be accessible from any PC in the network.

- Safe re-configuration of payload from any current state to any another state shall be available. On/off sequencing shall be taken care.

- GUI interface shall be there to show current state of payload with switching complexity to change from current payload configurations to new configurations.

- Provision to generate test procedures document.

- Provision to exclude ‘ON’ commands while configuring the payload.

- Provision to set configuration based on test parameter.

- Provision to include call files along with other commands while setting configuration.

- Provision to inhibit telecommands.

- Provision to limit the number of TWTAs /SSPAs to be switched ‘ON’ based on current limit and/or thermal requirement.

- Provision to keep adjacent channels at maximum BOA.

- Provision to check BAS current limit before generation of test procedure.

- Provision to generate incremental procedures files.

- Provision for configuring GISAT Series Data transmitter.

- Provision for test phase based commanding.

.

2.3 Constraints

As the Telecommand software has to work along with other software (COMPACSS/NAVPACSS/METPACSS and GIMPACSS). The software interfaces shall be compatible with existing software’s.

2.4 Assumptions and Dependencies

- “PAYLOAD_WIN” application should be running In SCC.

- IP address should be configured in Payload_win.inp file in SCC.

- SCC details (IP, Port Number, SCC login credentials, test folder path) should be entered in TM-TC database of SMART-TC.

2.5 Apportioning of requirements

The following requirements are planned for the next version of the software:

- Fully automated tele-commanding.

The above requirement of ‘Fully automated tele-commanding’ is addressed in version2.1.

# Specific Requirements

3.1 External Interface Requirements

### 3.1.1 Spacecraft Checkout Computer

“payload_win” application should be running In Spacecraft Checkout Computer and IP address should be configured in payload_win.inp file.

### 3.1.2 User Interfaces

- The user interface shall have a provision to select tele-commands from a searchable drop down list.

- The user interface shall have a provision to select BOA commands for a particular configuration from a searchable drop down list.

- The user interface shall have a provision to select tele-command Files from a searchable drop down list.

- The user interface shall have a provision to send data commands manually.

- The user interface shall have a provision to send configuration based commands.

- The user interface shall have a provision to show current state of payload system.

- The user interface shall have a provision to generate test procedures for all configurations.

### 3.1.3 Software Interfaces

#### 3.1.3.1 Interface with Database

The software shall connect to the database through ODBC. The TMTC database is used (Microsoft Access).All software interaction with the database will be via ODBC. Suitable ODBC names shall be given which can be used to interact with the databases.

3.2 Functional Requirements

### Database tables for Telecommands

From users perspective following are the database tables required to make this application database driven.

#### 3.2.1.1 Following is the database table for tele-commands.

This Database table contains all the tele-command mnemonics along with effected telemetry.

Table 3Telecommand Database Table

#### 3.2.1.2 Following is the database table for tele-command for configuration based BOA.

This table contains configuration based attenuation tele-commands.

Table 4 Configuration based BOA database Table

#### 3.2.1.3 Following is the database table for tele-command files

This table contains all payloadTC files existed in SCC. These files can be called from PCC for execution.

Table 5Telecommand files database table

#### Following is the database table for data commands

This table contains all the data-commands related to payload systems. A mapping table for all possible data values to user friendly name shall be defined (Table 7) to ease user selection from GUI.

Table 6 Data commands Database

q

Table 7 Data Commands Mapping Table

#### Following is the database table for configuration based commanding

This table contains all the tele-commands applicable for each configuration.

Table 8 Configuration based commanding database

#### Following is the database table for Spacecraft checkout computer details

Table 9 SCC Details

### 3.2.2 Provision to send tele-commands

The user shall have a provision to select one or multipletele-command mnemonics specified in TC column of database table (mentioned in section 3.2.1.1) from a dropdown list as shown in following image. After selection provision shall be there to generate tele-command test-procedure and trigger its execution.

Figure 2GUI for Tele-commands Drop down

### 3.2.3 Provision to trigger file execution.

The user shall have a provision to select any one tele-command File specified in Filename column of database table (mentioned in section 3.2.1.3) from a dropdown list as shown in following image.After selection provision shall be there to trigger its execution.

Figure 3 GUI for selecting Tele-command Files

### 3.2.4 Provisionto send configuration based BOA.

The user shall have provision to select one or multiple required BOA data command specified indatabase table (mentioned in section 3.2.1.2) from a dropdown list as shown in following image. After selection provision shall be there to generate tele-command test-procedure and trigger its execution.

Figure 4GUI for selecting BOA commands

### 3.2.5 Provision to send required BOA manually.

The user shall have a provision to select any one required data command and enter data part of the command manually or a drop down has to come if a mapping exists for the selected data-command . Data commands mnemonics will be specified in database table (mentioned in section 3.2.1.4) which are displayed in a dropdown list as shown in following image.After selection provision shall be there to generate tele-command test-procedure and trigger its execution.

Figure 5GUI for Sending Data commands

### 3.2.6 Provision to send configuration based commands.

The user shall have a provision to enter configuration number, to select only switch commands, to select BOA and to select test parameter as shown in following image. Software shallgenerate test procedure with proper sequencing. In case of TM data break it shall alert the user.

Figure 6GUI for sending configuration based commands

### 3.2.7 Search option to select commands or files

The user shall be able to search commands/file names from the dropdown list for all the cases shown above (section 3.2.2 to 3.2.6).

### 3.2.8 Provision should be there for command grouping

The user shall be able to select a commands group, based on this selection a dropdown list will be populated with commands corresponding to the group selected.

### 3.2.9Application should be accessible from any PC in the network

The application shall be accessible from any PC in the network. This is to allow user to work remotely.

### 3.2.10 Safe re-configuration of payload from any current state to any another state shall be available. On/off sequencing shall be taken care.

The user shall be able to safely re-configure payload from any current state to any other state. The software shall have intelligence for configuring payload. It shall take care of command on/off sequencing.

### 3.2.11GUI interface shall be there to show current state of payload with switching complexity to change from current payload configurations to new configurations.

User interface shall be there to check current payload status along with switching complexity to re-configure payload from current state to any other state.

### 3.2.12Provision shall be there to generate test procedures document.

The user shall be able to generate test procedures document for all payload configurations defined in database.

### 3.2.13Provision shall be there to exclude on commands while configuring the payload.

The user shall have a provision to exclude on commands in generated test procedure while re-configuring payload.

### 3.2.14Provision shall be there to set configuration based on test parameter.

The user shall have a provision to configure payload based on test-parameter, Based on select test parameter additional commands required for that test shall be issued.

### 3.2.15Provision shall be there to include call files along with other commands while setting configuration.

The user shall have an option to call any file/procedure   either resident at SCC or at PCC while generating the test procedure file. User can create a file and invoke it using Smart TC along with other commands of configuration.

### 3.2.16Provision shall be there to inhibit tele-commands.

The user shall have an option to inhibit tele-commands. During payload characterization there may be some elements of the payload which are either not mounted or failed. Such systems are prohibited from turning on. So, inhibit option to be included in Smart TC which will inhibit the particular system from being turned ON.

### 3.2.17Provision shall be there to limit the number of TWTAs /SSPAs to be switched on based on current limit and/or thermal requirement.

There shall be logic in Smart TC to limit the number of TWTAs /SSPAs to be switched on based on current limit and/or thermal requirement.

BAS current limit is set by the main frame team. Based on this limit, software should be able to limit maximum number of TWTs/SSPAs that can be turned ‘ON’ such that BAS current limit is not crossed. And similar logic to be provided to handle thermal constraint

### 3.2.18Provision shall be there to keep adjacent channels at maximum BOA.

During payload characterization, the standard practice involves putting adjacent channels at MAX BOA. Smart TC can take this into account and generate telecommands comprising of adjacent TWTA/SSPA at max BOA.

### 3.2.19 Provision to generate incremental procedure files

During payload reconfiguration, sequence of commands to be followed strictly.

For example TWTA can be turned on only if switches are configured.

Sample Procedure:

EXPECTED  twt1-sw-io-pos=pos13

SEND twt1-sw-io-pos13

EXPECTED  twt1-sts=on

SEND twt1-on

In above procedure, if expected for the first command fails then the rest of the commands should not be sent.

This approach creates problem for some kind payloads which uses multiplexed telemetry with different sampling time (More than one master frame 32sec).  As ACSS waits for 32sec before checking expected telemetry if sampling time is more than 32sec expected will fail before telemetry change happens.

To address this problem, the procedure can be split into multiples files without expected command, the telemetry check has to happen at PCC side for each command before generating the next command sequence. i.e for above sample procedure first command will be issued without expected value later after expected telemetry occurs next twta on command will is issued. The software has to wait for telemetry change based on its sampling time , if telemetry not changed after sampling time software has to alert user regarding anamoly and terminate the next configuration sequence.

3.2.20 Provision to cater GISAT Data transmitter Payload Test Requirement

- GISAT Data Transmitter contains BSU-M, BSU-R, 16 transmit module having 4 SSPAs each, DA-M, DA-R, LOUC-M, LOUC-R, DHE-M and DHE-R.

- To turn on GISAT payload procedure is explained below in flow chart

- Procedure to configure Configuration 1(MAIN CONFIG)

START

Configure BSU

Send BEPC-M-ON

Check BSU-FPGA-STS = Done

Send ThrComp-Dis

Phase-Comp-BPSS

Perform subsystem selection

send Ku-UC-DA-M-Sel

send QPSK-MOD-M-Sel

ISOLATErequired TX module

Call TX-MOD1-ON.tst

Send Jump-Default

Turn on the EPC

Send ALL-EPC-ON

A

G

O

Check Telemetry

KU1-4-EPC-STS = ON

DA-M-EPC-STS = ON

LOUC-M-EPC-STS= ON

A

SET DA GAIN AND PHASE

SEND PH-GAIN-Data-DA 0006

CALL ph4.tst

CALL ph5.tst

CALL phase6.tst

send PH-GAIN-DATA

Configure on board modulator

SEND DC-DC-M-SEL

SEND PLC-M-ON

SEND ATM-1-ON

SEND ATM-2-ON

Wait 128 sec

Check Telemetry

HSW-PLC-M-Sts = ON

D

Turn on Modulator

DHE-EPC-M-ON

WAIT  30 seconds

send MOD-FPGA-Rst

Wait 128 sec

Check Telemetry

PLCDHE-EPCM-Sts = ON

QPSK-MOD-M-Sts = Mod-ON

Turn off modulation  
Send QPSK-MOD-On-Of

Check Telemetry

QPSK-MOD-M-Sts = Mod-off

Perform Cw measurements

Turn on modulation *perform modulated mesaurements

E

F

E

Do u want to perform test  in same Tx module?

Check Telemetry

KU1-4-EPC-STS = ON

DA-M-EPC-STS = ON

LOUC-M-EPC-STS= ON

F

Call files for next cfg in same module

CALL ph4.tst

CALL ph5.tst

CALL phase6.tst

send PH-GAIN-DATA

G

Turn off the system

Send Dhe-epc-outoff

Send all-epc-of

Wait 128 sec

Check Telemetry

PLCDHE-EPCM-Sts = OFF

KU1-4-EPC-STS = OFF

DA-M-EPC-STS = OFF

LOUC-M-EPC-STS= OFF

Do u want to perform test  in next Tx module?

Do u want to changeover from main to redt?

H

STOP

Turn off the system

Send Dhe-epc-outoff

Send all-epc-of

SEND bepc-m-off

Configure BSU

Send BEPC-R-ON

Check BSU-FPGA-STS = Done

Send ThrComp-Dis

Phase-Comp-BPSS

Perform subsystem selection

send Ku-UC-DA-R-Sel

send QPSK-MOD-R-Sel

ISOLATE required TX module

Call TX-MOD1-ON.tst

Turn on the EPC

Send ALL-EPC-ON

Send Jump-Default

i

N

H

Procedure to configure Configuration 73(REDT CONFIG)

Check Telemetry

KU1-4-EPC-STS = ON

DA-R-EPC-STS = ON

LOUC-R-EPC-STS= ON

i

SET DA GAIN AND PHASE

SEND PH-GAIN-Data-DA 0006

CALL ph4.tst

CALL ph5.tst

CALL phase6.tst

send PH-GAIN-DATA

Configure on board modulator

SEND DC-DC-R-SEL

SEND PLC-R-ON

SEND ATM-1-ON

SEND ATM-2-ON

Wait 128 sec

Check Telemetry

HSW-PLC-R-Sts = ON

K

Turn on Modulator

DHE-EPC-R-ON

WAIT  30 seconds

send MOD-FPGA-Rst

Wait 128 sec

Check Telemetry

PLCDHE-EPCR-Sts = ON

QPSK-MOD-R-Sts = Mod-oN

Turn off modulation  
Send QPSK-MOD-On-Of

Check Telemetry

QPSK-MOD-R-Sts = Mod-off

Perform Cw measurements

Turn on modulation *perform modulated mesaurements

L

M

L

Do u want to perform test  in same Tx module?

Check Telemetry

KU1-4-EPC-STS = ON

DA-R-EPC-STS = ON

LOUC-R-EPC-STS= ON

M

Call files for next cfg in same module

CALL ph4.tst

CALL ph5.tst

CALL phase6.tst

send PH-GAIN-DATA

N

Turn off the system

Send Dhe-epc-outoff

Send all-epc-of

Wait 128 sec

Check Telemetry

PLCDHE-EPCR-Sts = OFF

KU1-4-EPC-STS = OFF

DA-R-EPC-STS = OFF

LOUC-R-EPC-STS= OFF

Do u want to perform test  in next Tx module?

Do u want to changeover from REDT  to MAIN?

O

STOP

Turn off the system

Send Dhe-epc-outoff

Send all-epc-of

SEND bepc-r-off

- WITH SINGLE TRANSMIT MODULE ON.

CW Measurement = Power, Frequency, spurious

Modulated measurement = Channel Power, RF Spectrum plot

STEP-1: For measurement of CFG-1 Following set of Commands need to be sent.

expected BSU-EPC-M-Sts = ON

send BEPC-M-ON( To turn on main Beam steering unit)

Common for cfg 1-64

Expected BSU-FPGA-STS = Done

Send Jump-Default (FPGA will jump at default address)

SendThrComp-Dis // During Th-VAC send  ThrComp-Ena

Send Phase-Comp-BPSS( To bypass onboard lookup table of phase and gain)

expected LOUC-SEL-STS = Main

send Ku-UC-DA-M-Sel( To select main upconverter and DA)

expected MOD-SEL-STS = Main

send QPSK-MOD-M-Sel (To select main modulator)

// This call file is used to select Single Transmit Module

CALL TX-MOD1-ON.tst ( This file contains details of TX module to be selected                      to turn on)

expected KU1-4-EPC-STS = ON;

DA-M-EPC-STS = ON;

LOUC-M-EPC-STS= ON

send ALL-EPC-ON( it will turn on EPCs of all selected TX module, DA and LOUC)

SEND PH-GAIN-Data-DA 0006

CALL 106-2-c1.tst (This file contains details of BOA and PHASE of SSPA’s 1-32)

CALL ph5-00-3364.tst (This file contains details of BOA and PHASE of SSPA’s 33-64)

CALL phase6.tst (common file to be send for all configuration after above two    files)

send PH-GAIN-DATA (To apply BOA and PHASE data)

SEND DC-DC-M-SEL (main DC-DC supply selection)

EXPECTED HSW-PLC-M-Sts = ON// from here onwards telemetry will update in 128 sec approx.

SEND PLC-M-ON (To turn on payload controller)

EXPECTED ATM-1-Sts = ON

SEND ATM-1-ON (To turn on analog telemetry module-1)

EXPECTED ATM-2-Sts = ON

SEND ATM-2-ON (To turn on analog telemetry module-2)

EXPECTED PLCDHE-EPCM-Sts = ON

SEND DHE-EPC-M-ON ( to turn on main data handling electronics)

Wait 30 sec

Check QPSK-MOD-M-Sts = Mod-on

If mod is on then proceed with second step

Step-2 : Perform all the modulated measurement here( Channel Power, RF SPECTRUM PLOT)

Step-3 :After this for CW Measurement send following Command

ExpectedQPSK-MOD-M-Sts = Mod-off

send QPSK-MOD-On-Of

perform all CW measurement here

3.2.20.1 Details of TX module file

Tx-MOD1ON file

It consist details of Tx module to be selected to turn on as follows

000.001 select segid c8 - FIXED

000.002 select cmdmodemsgtable  - FIXED

000.003 sendlistData_word  0001 ; ! No of Patch pkts! - FIXED

TX module 1 is selected to turn on

Data_word  6051 ; ! CMD WORD1 ! - FIXED

Data_word  0000 ; !DATA WORDS!

Data_word  0019 ;

Data_word  0019 ;

Data_word  0019 ;

Data_word  0019 ;

Data_word  0019 ;

Data_word  0019 ;

TX module 2-16 are isolated

Data_word  0019 ;

Data_word  0019 ;

Data_word  0019 ;

Data_word  0019 ;

Data_word  0019 ;

Data_word  0019 ;

Data_word  0019 ;

Data_word  0019 ;

Data_word  0019 ;

Data_word  0055

000.004 select    cmdmode normal

000.005 singlestep

000.006 end

- Data word 0000 is for selection and data word 0019 is for isolation

- Multiple TX module selection for Turn-on  is possible by giving multiple data word 0000 on respective locations based on TX module number

3.2.20.2 Details of BOA and PHASE file for SSPA 1-32

000.001 select segid c8- FIXED

000.002 select cmdmodemsgtable- FIXED

000.003 sendlistData_word  0001 ;- FIXED

Data_word  6080 ; ! CMD WORD1 !- FIXED

BOA and PHASE details of SSPA 1

Data_word  0413 ; !DATA WORDS!

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

BOA and PHASE details of SSPA 2-32

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00

000.004 select   cmdmode normal- FIXED

000.005 singlestep- FIXED

000.006 end- FIXED

- Data word 0413  BOA and PHASE is assigned to SSPA-1

- DATA word 1F00 maximum BOA and 0 PHASE is assigned to all other 2-32 SSPAs

- Above example is for configuration number-1 (SSPA-1)

- In configuration defined BOA and PHASE need to send for respective SSPA and rest SSPAs will be at 1F00.

3.2.20.3 Details of BOA and PHASE file for SSPA 33-64 file

000.001 select segid c8- FIXED

000.002 select cmdmodemsgtable- FIXED

000.003 sendlistData_word  0001 ;- FIXED

Data_word  60A0 ; ! CMD WORD1 !- FIXED

BOA and PHASE details of SSPA 33

Data_word  0413 ; !DATA WORDS!

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

BOA and PHASE details of SSPA 33-64

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00 ;

Data_word  1F00

000.004 select   cmdmode normal- FIXED

000.005 singlestep- FIXED

000.006 end- FIXED

- Data word 0413  BOA and PHASE is assigned to SSPA-33

- DATA word 1F00 maximum BOA and 0 PHASE is assigned to all other 34-64 SSPAs

- Above example is for configuration number-33(SSPA-33)

- In configuration defined BOA and PHASE need to send for respective SSPA and rest SSPAs will be at 1F00.

3.2.20.4 Details of common file need to send with above two files

000.001 select segid c8

Fixed for all configuration

000.002 select cmdmodemsgtable

000.003 sendlistData_word  0001 ;

Data_word  60C0 ; ! CMD WORD1 !

Data_word  0055

000.004 select   cmdmode normal

000.005 singlestep

000.006 end

For Changover FROM CFG-1 to CFG-2

Step1: Send following set of files to changeover from CFG1 to CFG2

CALL 90-1-c2.tst

CALL ph5-00-3364.tst

CALL phase6.tst

Step 2: Send following set of commands

send PH-GAIN-DATA

Check QPSK-MOD-M-Sts = Mod-on

Step-3 : Perform all the modulated measurement here( Channel Power, RF SPECTRUM PLOT)

Step-4 :After this for CW Measurement send following Command

ExpectedQPSK-MOD-M-Sts = Mod-off

send QPSK-MOD-On-Of

perform all CW measurement here

For Changover FROM CFG-2 to CFG-3

Step1: Send following set of files to changeover from CFG2 to CFG3

CALL 90-1-c4.tst

CALL ph5-00-3364.tst

CALL phase6.tst

Step 2: Send following set of commands

send PH-GAIN-DATA

Check QPSK-MOD-M-Sts = Mod-on

Step-3 : Perform all the modulated measurement here( Channel Power, RF SPECTRUM PLOT)

Step-4 :After this for CW Measurement send following Command

ExpectedQPSK-MOD-M-Sts = Mod-off

send QPSK-MOD-On-Of

perform all CW measurement here

For Changover FROM CFG-3 to CFG-4

Step1: Send following set of files to changeover from CFG3 to CFG4

CALL 90-1-c4.tst

CALL ph5-00-3364.tst

CALL phase6.tst

Step 2: Send following set of commands

send PH-GAIN-DATA

Check QPSK-MOD-M-Sts = Mod-on

Step-3 : Perform all the modulated measurement here( Channel Power, RF SPECTRUM PLOT)

Step-4 :After this for CW Measurement send following Command

ExpectedQPSK-MOD-M-Sts = Mod-off

send QPSK-MOD-On-Of

perform all CW measurement here

For Changover FROM CFG-4 to CFG-5(IF one Tx module is on)

In CFG-5 changeover from transmit module 1 (TX-MOD1) to transmit module 2(TX-MOD2) is done. Following are the set of commands needs to be send.

Step1: Put off DHE-EPC, U/C Main , DA Main and TX-MOD1 (contains SSPA1 to SSPA4).

EXPECTED PLCDHE-EPCM-Sts= OF

sendDHE-EPC-OutOFF

expectedKU1-4-EPC-STS = OFF;

KU5-8-EPC-STS = OFF;

KU1-4-EPC-STS = OFF;

KU5-8-EPC-STS = OFF;

KU9-12-EPC-STS = OFF;

KU13-16-EPC-STS = OFF;

KU17-20-EPC-STS = OFF;

KU21-24-EPC-STS = OFF;

KU25-28-EPC-STS = OFF;

KU29-32-EPC-STS = OFF;

KU33-36-EPC-STS = OFF;

KU37-40-EPC-STS = OFF;

KU41-44-EPC-STS = OFF;

KU45-48-EPC-STS = OFF;

KU49-52-EPC-STS = OFF;

KU53-56-EPC-STS = OFF;

KU57-60-EPC-STS = OFF;

KU61-64-EPC-STS = OFF;

DA-M-EPC-STS = OFF;

LOUC-M-EPC-STS= OFF

sendALL-EPC-OF

Step2:   Select Transmit module2(TX-MOD2,contains SSPA5 to SSPA8) and proceed with following commanding.

CALL TX-MOD2-ON.tst

expected KU5-8-EPC-STS = ON;

DA-M-EPC-STS = ON;

LOUC-M-EPC-STS= ON

send ALL-EPC-ON

SEND PH-GAIN-Data-DA 0006

CALL 106-2-c5.tst

CALL ph5-00-3364.tst

CALL phase6.tst

send PH-GAIN-DATA

EXPECTED PLCDHE-EPCM-Sts = ON

SEND DHE-EPC-M-ON

Wait 30 sec

Check QPSK-MOD-M-Sts = Mod-on

If mod is on then proceed with third step

Step-3 : Perform all the modulated measurement here( Channel Power, RF SPECTRUM PLOT)

Step-4 :After this for CW Measurement send following Command

ExpectedQPSK-MOD-M-Sts = Mod-off

send QPSK-MOD-On-Of

perform all CW measurement here.

- For   CFG 6 to 8 (SSPA 6 to SSPA 8) follow the procedure of Change over like CFG 1 to 2, 2 to 3 and CFG 3 to 4.

- For next Transmit module 3(TX-MOD3) onwards follow the same procedure as above mentioned.

- WITH ALL TRANSMIT MODULE ON.

CW Measurement = Power, Frequency, spurious

Modulated measurement = Channel Power, RF Spectrum plot

STEP-1: For measurement of CFG-1 Following set of Commands need to be sent.

expected BSU-EPC-M-Sts = ON

send BEPC-M-ON

Wait 7 sec

Expected BSU-FPGA-STS = Done

Send Jump-Default

SendThrComp-Dis // During Th-VAC send  ThrComp-Ena

Send Phase-Comp-BPSS

expected LOUC-SEL-STS = Main

send Ku-UC-DA-M-Sel

expected MOD-SEL-STS = Main

send QPSK-MOD-M-Sel

// This call file is used to select ALL Transmit Module

CALL TX-ALL-ON.tst

expected KU1-4-EPC-STS = ON;

KU5-8-EPC-STS = ON;

KU1-4-EPC-STS = ON;

KU5-8-EPC-STS = ON;

KU9-12-EPC-STS = ON;

KU13-16-EPC-STS = ON;

KU17-20-EPC-STS = ON;

KU21-24-EPC-STS = ON;

KU25-28-EPC-STS = ON;

KU29-32-EPC-STS = ON;

KU33-36-EPC-STS = ON;

KU37-40-EPC-STS = ON;

KU41-44-EPC-STS = ON;

KU45-48-EPC-STS = ON;

KU49-52-EPC-STS = ON;

KU53-56-EPC-STS = ON;

KU57-60-EPC-STS = ON;

KU61-64-EPC-STS = ON;

DA-M-EPC-STS = ON;

LOUC-M-EPC-STS= ON

send ALL-EPC-ON

SEND PH-GAIN-Data-DA 0006

CALL 106-2-c1.tst

CALL ph5-00-3364.tst

CALL phase6.tst

send PH-GAIN-DATA

SEND DC-DC-M-SEL

EXPECTED HSW-PLC-M-Sts = ON// from here onwards telemetry will update in 128 sec approx.

SEND PLC-M-ON

EXPECTED ATM-1-Sts = ON

SEND ATM-1-ON

EXPECTED ATM-2-Sts = ON

SEND ATM-2-ON

EXPECTED PLCDHE-EPCM-Sts = ON

SEND DHE-EPC-M-ON

Wait 30 sec

Check QPSK-MOD-M-Sts = Mod-on

If mod is on then proceed with second step

Step-2 : Perform all the modulated measurement here( Channel Power, RF SPECTRUM PLOT)

Step-3 :After this for CW Measurement send following Command

ExpectedQPSK-MOD-M-Sts = Mod-off

send QPSK-MOD-On-Of

perform all CW measurement here

For Changover FROM CFG-1 to CFG-2

Step1: Send following set of files to changeover from CFG1 to CFG2

CALL 90-1-c2.tst

CALL ph5-00-3364.tst

CALL phase6.tst

Step 2: Send following set of commands

send PH-GAIN-DATA

Check QPSK-MOD-M-Sts = Mod-on

Step-3 : Perform all the modulated measurement here( Channel Power, RF SPECTRUM PLOT)

Step-4 :After this for CW Measurement send following Command

ExpectedQPSK-MOD-M-Sts = Mod-off

send QPSK-MOD-On-Of

perform all CW measurement here

- Same procedure has to be followed for all 64 main combinations only respective call files for test cfg needs to be given.

For Changover FROM MAIN CFG’s to REDUNADANT CFG’s:

Main Configuration to redundant Configuration changeover, Following are the set of commands needs to be send.

Step1: Put off DHE-EPC, U/C Main, DA Main, Transmit Module(TX-MOD) and BSU Main.

EXPECTED PLCDHE-EPCM-Sts= OF

sendDHE-EPC-OutOFF

expectedKU1-4-EPC-STS = OFF;

KU5-8-EPC-STS = OFF;

KU1-4-EPC-STS = OFF;

KU5-8-EPC-STS = OFF;

KU9-12-EPC-STS = OFF;

KU13-16-EPC-STS = OFF;

KU17-20-EPC-STS = OFF;

KU21-24-EPC-STS = OFF;

KU25-28-EPC-STS = OFF;

KU29-32-EPC-STS = OFF;

KU33-36-EPC-STS = OFF;

KU37-40-EPC-STS = OFF;

KU41-44-EPC-STS = OFF;

KU45-48-EPC-STS = OFF;

KU49-52-EPC-STS = OFF;

KU53-56-EPC-STS = OFF;

KU57-60-EPC-STS = OFF;

KU61-64-EPC-STS = OFF;

DA-M-EPC-STS = OFF;

LOUC-M-EPC-STS= OFF

sendALL-EPC-OF

SendThrComp-Dis

expected BSU-EPC-M-Sts = OFF

send BEPC-M-OFF

send atmboth-0ff

send plc-off

STEP-2: For measurement of CFG-73 Following set of Commands need to be sent.

expected BSU-EPC-R-Sts = ON

send BEPC-R-ON

Wait 7 sec

Expected BSU-FPGA-STS = Done

Send Jump-Default

SendThrComp-Dis // During Th-VAC send  ThrComp-Ena

Send Phase-Comp-BPSS

expected LOUC-SEL-STS = Redt

send Ku-UC-DA-R-Sel

expected MOD-SEL-STS = Redt

send QPSK-MOD-R-Sel

Step 3:   Select Transmit module1(TX-MOD1, contains SSPA1 to SSPA4) and proceed with following commanding.

CALL TX-MOD1-ON.tst

expected KU1-4-EPC-STS = ON;

DA-R-EPC-STS = ON;

LOUC-R-EPC-STS= ON

send ALL-EPC-ON

SEND PH-GAIN-Data-DA 0006

CALL 106-2-c73.tst

CALL ph5-00-3364.tst

CALL phase6.tst

send PH-GAIN-DATA

SEND DC-DC-R-SEL

EXPECTED HSW-PLC-R-Sts = ON// from here onwards telemetry will update in 128 sec approx.

SEND PLC-R-ON

EXPECTED ATM-1-Sts = ON

SEND ATM-1-ON

EXPECTED ATM-2-Sts = ON

SEND ATM-2-ON

EXPECTED PLCDHE-EPCR-Sts = ON

SEND DHE-EPC-R-ON

Wait 30 sec

Check QPSK-MOD-R-Sts = Mod-on

If mod is on then proceed with second step

Step-2 : Perform all the modulated measurement here( Channel Power, RF SPECTRUM PLOT)

Step-3 :After this for CW Measurement send following Command

Expected QPSK-MOD-R-Sts = Mod-off

send QPSK-MOD-On-Of

perform all CW measurement here.

For redundant 64 cfg follow the same procedure as per the main cfg’s.

3.2.21 Provision for test phase based commanding.

The user shall have an optiontosend telecommands based on test phases.

Examples-

- Thr-comp-dis command need to be send in clean room IST phase

- Thr-comp-ena command need to be send in TVAC IST phase

3.3. Performance Requirements

- This package shall be able to run on a PC with RAM of 1 GB along with other packages of COMPACSS.

3.4. Design Constraints

- The software interfaces are fixed as the interface at SCC should not get affected.

3.5. Software System attributes

Reliability Software reliability is the probability of failure free software operation for a specified period of time in a specified environment. To increase the reliability aspects, this software shall undergo proper verification & validation with exhaustive test cases to cover all possibilities of errors

Availability Proper and systematic back-up and recovery features to be planned to increase the availability of the software

Maintainability The software shall follow data driven approach to cater to easy maintenance across different projects and varying requirements in the future

Tele-command software Goals:

- To configure the Payload as per user/test requirements by generation of telecommand(s) files dynamically in CCL-compliant format from PCC &placing requests to SCC forexecution of the same.

- Payload Safety shall be ensured throughout the process of commanding / re-configuration of the payload.

- Commanding/re-configuration of the payload shall take the minimum possible time

- Shall be user-friendly

# Comparison of Featuresof all versions of TelecommandApplication

| Comparison of Features |  |  |  |  |
| --- | --- | --- | --- | --- |
| Feature | V1.0 | V2.0 | V2.1 (SMART TC) | Remarks |
| SendingTelecommands | Y | Y | Y |  |
| SendingTelecommand Files | Y | Y | Y |  |
| Sequential Switch Commanding | Y (Macros are required) | Y | Y |  |
| Sending Configuration based BOA | N | Y | Y |  |
| Manual data commanding | N | Y | Y |  |
| Configuration based commanding | N | Y | Y |  |
| Incremental commanding | N | Y | N | This feature not required now. |
| Commands Grouping | N | Y | Y |  |
| Commands Search Capability | N | Y | Y |  |
| VEPW Commanding | N | Y | Y |  |
| Commanding Inelegance(Safe re-configuration of payload from any current state to any other state) | N | N | Y |  |
| Test parameter based commanding | N | N | Y |  |
| Tele-Command Inhibit Option | N | N | Y |  |
| Call files along with configuration based commands | N | N | Y |  |
| Display of current state of payload with switching complexity. | N | N | Y |  |
| Generation of test procedures document | N | N | Y |  |
| Exclude ‘ON’commands while configuring the payload | N | N | Y |  |
| Limiting the number of TWTAs /SSPAs to be switched ‘ON’ based on current limit and/or thermal requirement | N | N | Y |  |
| Provision to keep adjacent channels at maximum BOA | N | N | Y |  |

Table 11Features Comparison

# Requirements Traceability Matrix

| Software Requirement Id | Section | System Requirement Id | Section | Description |
| --- | --- | --- | --- | --- |
| RID 1 | 3.2.1 |  |  | Database tables for tele-commands |
| RID 2 | 3.2.2 |  |  | Provision should be there to send tele-commands. |
| RID 3 | 3.2.3 |  |  | Provision should be there to trigger file execution. |
| RID 4 | 3.2.4 |  |  | Provision should be there to send configuration based BOA. |
| RID 5 | 3.2.5 |  |  | Provision should be there to send required BOA manually. |
| RID 6 | 3.2.6 |  |  | Provision should be there to send configuration based commands. |
| RID 7 | 3.2.7 |  |  | Search option should be there to select commands or files. |
| RID 8 | 3.2.8 |  |  | Commands grouping option should be available. |
| RID 9 | 3.2.9 |  |  | Application should be accessible from any PC in the network. |
| RID 10 | 3.2.9 |  |  | Application should be accessible from any PC in the network. |
| RID 11 | 3.2.10 |  |  | Safe re-configuration of payload from any current state to any another state shall be available. On/off sequencing shall be taken care |
| RID 12 | 3.2.11 |  |  | GUI interface shall be there to show current state of payload with switching complexity to change from current payload configurations to new configurations |
| RID 13 | 3.2.12 |  |  | Provision shall be there to generate test procedures document. |
| RID 14 | 3.2.13 |  |  | Provision shall be there to exclude on commands while configuring the payload |
| RID 15 | 3.2.14 |  |  | Provision shall be there to set configuration based on test parameter |
| RID 16 | 3.2.15 |  |  | Provision shall be there to include call files along with other commands while setting configuration |
| RID 17 | 3.2.16 |  |  | Provision shall be there to inhibit tele-commands. |
| RID 18 | 3.2.17 |  |  | Provision shall be there to limit the number of TWTAs /SSPAs to be switched on based on current limit and/or thermal requirement. |
| RID 18 | 3.2.18 |  |  | Provision shall be there to keep adjacent channels at maximum BOA |

Table 10Requirements Traceability